# Skynet Build: 2026-09-30 (power plugs, battery automation, node updates)

Homelab cluster skynet · jameshomelab.org · Tasmota plugs .72/.73/.74 · AdGuard + Deco BE67 reservations · wrk-03 to 26.04.1 · rolling reboots

Status: complete 2026-09-30 21:02 CT · 6/6 nodes Ready · Ubuntu 26.04.1 LTS · kernel 7.0.0-34 · k3s v1.36.3+k3s1 · Deco reservations 24 · CNPG primary homelab-postgres-1 (cp-02)

# 1. Purpose

This document records the homelab work done on Wednesday 2026-09-30 (all times CT). It covers two tracks: (1) smart-plug power control for the two battery-powered machines (the M5 MacBook Pro and the MacBookPro8,3 worker skynet-wrk-02), plus a third spare plug, with DNS names, DHCP reservations, battery automation, failure alerting and an SMS status command; and (2) cluster node maintenance: a wrk-02 apt upgrade, the skynet-wrk-03 release upgrade from Ubuntu 24.04.5 to 26.04.1 Desktop, and rolling reboots of cp-02, wrk-02 and cp-03 to clear pending reboot-required flags.

Design rule  Never force anything. Drains run without --force, any PDB or drain block stops the sequence for review, nodes go one at a time with a verified Ready state and healthy etcd/Longhorn before the next, and every router write is read, add one, read again, diff. Passwords never appear on a command line or in logs; in this document every secret is a placeholder (the Tasmota web password is shown as <plug-password>).

Sources: /workspace/build-log/2026-09-30-*.md (plugs-adguard, plugs-deco, plugs-auxpower, wrk03-upgrade, node-reboots), the box copies of the battery scripts (/workspace/m5/battery_alert.applescript, /workspace/bp/battery-plug.{sh,service,timer}, /workspace/bbtest.sh), the n8n router config and parser tests under /home/box/agent-data/shared, and the skills deco-dhcp-address-reservations-local-api and n8n-sms-commands. Anything the sources do not cover is listed in Appendix B.

# 2. Summary of outcomes

| Area | Outcome |
| --- | --- |
| SkynetM5Power (.74) | Feeds the M5 charger. M5 BatteryAlert turns it On at ≤20% on battery and Off at ≥80% on AC; AdGuard name skynetm5power; Deco reservation added |
| Skynet-wrk-02-Power (.72) | Feeds wrk-02's charger. battery-plug.timer on wrk-02 switches it at 20/80%; alerts only on failure; AdGuard name skynet-wrk-02-power; Deco reservation added |
| AuxPower (.73) | New plug. Tasmota 8.5.0 → 15.6.0 via step path, config mirrors .72/.74, AdGuard name auxpower, Deco reservation added. Not monitored |
| cmd show battery | SMS command answered locally (not by n8n) with M5 and wrk-02 battery + plug state |
| wrk-02 no-sleep | logind lid/idle actions set to ignore; confirmed unchanged after reboot |
| Deco BE67 | Reservations added through the hidden local API: 21 → 24. 192.168.68.40 (kube-vip) left unreserved |
| wrk-02 apt | 0 upgraded; 4 glycin packages held back by phased rollout (left alone) |
| skynet-wrk-03 | Drained, backed up, network fixed, k3s node-ip pinned, upgraded 24.04.5 → 26.04.1 LTS + ubuntu-desktop, PHP 8.3 → 8.5 restored for nginx, uncordoned |
| Rolling reboots | cp-02, wrk-02, cp-03 rebooted one at a time after an etcd snapshot; VIP and etcd leader moved cleanly; CNPG switchover to homelab-postgres-1 (cp-02) before cp-03 |
| Final cluster | 6/6 Ready, all Ubuntu 26.04.1 LTS, kernel 7.0.0-34-generic, k3s v1.36.3+k3s1, containerd 2.3.2-k3s2, no reboot-required anywhere |

# 3. Environment

| Item | Value |
| --- | --- |
| Admin host for LAN work | Mac mini skynet (macOS user skynet, machine 4de696b8…); kubectl context default (targets the VIP) |
| Agent box | Holds secrets as env vars only ($ADGUARD_PASSWORD, $DECO_ADMIN_PASSWORD); cannot reach the LAN |
| Router | TP-Link Deco BE67 ×3, hw 1.0, fw 1.3.6 Build 26080612 Rel. 28671. Main Bedroom (master) 192.168.68.1, LAN MAC 10-5A-95-98-E9-52, mask 255.255.252.0; Kitchen 192.168.71.250; Office 192.168.71.249 |
| DNS | AdGuard Home 192.168.68.151, port 80, user cyberstar; bare-name rewrites; AdGuard DHCP off |
| Cluster | k3s skynet, API VIP 192.168.68.40:6443 (kube-vip); cp-01 .101, cp-02 .102, cp-03 .103, wrk-01 .111, wrk-02 .112, wrk-03 .113 |
| etcd | 3 members: skynet-cp-01-28ce3c6a, skynet-cp-02-8e2f2823, skynet-cp-03-bcffc880; etcd 3.6.14 |
| Postgres | CNPG cluster database/homelab-postgres, 3 instances, operator cloudnative-pg 1.30.0, image postgresql:18.4-standard-trixie |
| iMessage relay | BlueBubbles on the Mac mini, http://192.168.68.70:1234 (used by the wrk-02 failure alerts) |

# 4. Power plugs

Three Tasmota smart plugs are on the LAN. Two feed the chargers of the battery-powered machines so the batteries are held between 20% and 80% instead of sitting on a charger at 100%. The third (AuxPower) was set up the same way and is not tied to any automation.

| Plug | IP / DNS name | MAC | Tasmota hostname | Firmware | Role |
| --- | --- | --- | --- | --- | --- |
| SkynetM5Power | 192.168.68.74 / skynetm5power | 4C:11:AE:00:5E:86 | tasmota-005E86-7814 | Not recorded in the logs | M5 MacBook Pro charger |
| Skynet-wrk-02-Power | 192.168.68.72 / skynet-wrk-02-power | 4C:11:AE:00:64:E2 | tasmota-0064E2-1250 | 15.6.0(release-tasmota), ESP8266EX | skynet-wrk-02 charger |
| AuxPower | 192.168.68.73 / auxpower | C8:2B:96:07:00:F9 | tasmota-0700F9-0249 | 15.6.0(release-tasmota), Core 2.7.8, ESP8266EX 1MB | Spare; not monitored |

.72 and .74 report gateway 192.168.68.1, mask 255.255.252.0, DNS 192.168.68.151 (Tasmota Status 5).

## 4.1 Shared Tasmota configuration

Read from .72 and .74 at 20:35 and applied to .73 at 21:00. All three now match (the SetOption words on .73 are identical to .72).

| Setting | Value | Notes |
| --- | --- | --- |
| Timezone | 99 | Use the TimeDst/TimeStd rules below |
| TimeDst | 0,2,3,1,2,-300 | US Central daylight time |
| TimeStd | 0,1,11,1,2,-360 | US Central standard time |
| Latitude / Longitude | 29.719999 / -98.070000 | Set as 29.72 / -98.07 |
| SetOption128 | ON (1) | Lets /cm HTTP API calls through without a same-device Referer (the 15.6.0 default of 0 drops them) |
| PowerOnState | 3 |  |
| DeviceName / FriendlyName1 | Plug name | SkynetM5Power, Skynet-wrk-02-Power, AuxPower |
| WebPassword | <plug-password> | Web/API user admin; unauthenticated /cm calls answer "Need user=...&password=..." |
| OtaUrl | http://ota.tasmota.com/tasmota/release-15.6.0/tasmota.bin.gz | Set on .73 to match .72 |

## 4.2 SkynetM5Power and the M5 BatteryAlert

BatteryAlert is an AppleScript stay-open applet on the M5 (box copy /workspace/m5/battery_alert.applescript, dated 16:43). Every 60 seconds it reads the battery with pmset -g batt and drives SkynetM5Power over the Tasmota HTTP API (http://192.168.68.74/cm?user=admin&password=<plug-password>&cmnd=…).

| Condition | Action |
| --- | --- |
| ≤ 20% and on battery | Plug Power On; resent every minute until the plug replies POWER ON |
| ≥ 80% and on AC | Plug Power Off; resent every minute until the plug replies POWER OFF |
| 21–79% | Re-arm: clears the last alert and the confirmed plug state so the next crossing always sends a command |
| ≤ 21% on battery | One-time pre-alert ("plug the M5 charger into SkynetM5Power now") |
| Plug command fails | Logged; one macOS notification per failure streak ("Plug unreachable … Retrying every 60s"), then silent retries |
| Applet reopened | Status dialog: battery bar, %, power source, charge state, time remaining, live plug state, thresholds |

Every plug action (OK or FAILED, with the battery % and the plug's reply) is appended to ~/Scripts/battery_alert.log on the M5.

CRITICAL — M5 BatteryAlert still messages at every threshold crossing
The box copy still raises a macOS notification and an iMessage on low, pre-alert, and high crossings. Failure-only behavior is not verified for the live M5 copy; confirm it before treating the crossing messages as removed.

## 4.3 Skynet-wrk-02-Power and the wrk-02 battery-plug watcher

skynet-wrk-02 is a MacBookPro8,3 running as a k3s worker on battery. /usr/local/bin/battery-plug.sh (box copy /workspace/bp, dated 18:06–18:13) keeps its battery between 20% and 80% by switching Skynet-wrk-02-Power. It runs as a oneshot service from a systemd timer.

| Unit / file | Content |
| --- | --- |
| battery-plug.service | Description "Battery charge manager (Tasmota plug Skynet-wrk-02-Power)"; After/Wants network-online.target; Type=oneshot; ExecStart=/usr/local/bin/battery-plug.sh |
| battery-plug.timer | OnBootSec=1min, OnUnitActiveSec=60s, AccuracySec=5s, Persistent=true, WantedBy=timers.target |
| /etc/battery-plug.env | BLUEBUBBLES_PASSWORD=<redacted>, BB_URLS (parsed, not sourced) |
| /var/lib/battery-plug/state | desired, confirmed, sent_ts, pending, alerted, low_alarm (key=value); lock file for flock |
| /var/log/battery-plug.log | One line per decision, switch, verification and alert |

Logic on each run:

Read /sys/class/power_supply/BAT0/capacity and ADP1/online (a capacity above 100 is treated as 100, a calibration quirk). An unreadable battery logs an ERROR and exits.

On battery and ≤ 20% → want ON. On AC and ≥ 80% → want OFF. Otherwise do nothing.

Query the plug's actual state first; if it is already in the wanted state, just record it (no switch).

Otherwise send Power On/Off, retry once after 5 s, and require the plug to echo the wanted state.

Verify the switch really changed AC: ADP1/online must flip within 300 s.

Critical alarm once per discharge cycle if still on battery at ≤ 15%.

Alerts only on failure. Normal switching is logged but sends nothing. The three alert cases:

Switch failed: the plug did not confirm the command ("failed to switch plug On at N% … Retrying every minute."). Sent once per attempt streak; the switch is retried every run.

Verify failed: the plug confirmed but ADP1/online did not change within 300 s ("Charger may not be on this plug.").

Critical: ≤ 15% and still on battery ("Charger not restoring power?").

Alerts go to logger (user.crit, tag battery-plug) and as an iMessage through BlueBubbles on the Mac mini (POST /api/v1/message/text; the password is URL-encoded and passed with curl --config so it never shows in the process list). If no BlueBubbles URL answers HTTP 200, the script falls back to wall + syslog. /workspace/bbtest.sh is the one-off test of that alert path ("Test: wrk-02 battery-plug alert path works" to http://192.168.68.70:1234).

Verified at 20:21 (before) and 20:36 (after the wrk-02 reboot): battery-plug.timer active, running every minute, last run 20:35:28, battery-plug.service "Deactivated successfully".

### 4.3.1 wrk-02 battery cycle watchdog

The cycle watchdog was installed on wrk-02 at about 21:10 to measure real battery cycles separately from the existing charger switcher. It runs as a oneshot every 60 seconds (OnBootSec=90s), logs five-minute readings and events, and keeps completed-leg summaries and JSON state.

| Aspect | Installed / observed |
| --- | --- |
| Installed | /usr/local/bin/battery-cycle-watchdog; battery-cycle-watchdog.service and .timer; /etc/battery-cycle-watchdog.conf; monthly compressed logrotate (24 copies, su root syslog) |
| Data | /var/log/battery-cycles.csv, /var/log/battery-cycle-legs.csv, /var/lib/battery-cycle-watchdog/state.json; source copies in /workspace/wrk02-watchdog/ |
| Cycle thresholds | Baseline is the median of the first 3 valid legs. Alert at a 25% shrink (last 3 mean ≤75% of baseline), any 80→20 discharge under 40 minutes, or equivalent full-cycle shrink; 24-hour cooldown. |
| Temperature | Alert at BAT0 ≥45 °C; re-arm below 42 °C; 24-hour cooldown. |
| Alert path | BlueBubbles iMessage to the existing wrk-02 alert chat, prefix "wrk-02 battery-watchdog:"; if no URL returns HTTP 200, fallback to wall + syslog. Normal readings and switching send no message; no real watchdog alert was sent during this build. |

CRITICAL — wrk-02 battery holds only about 40% of rated capacity
The observed 80→20 discharge integrated about 35–40 Wh versus the 95 Wh rated pack (charge-side estimates are higher and the gauge counters conflict). Treat the pack as badly degraded until replaced or operated on AC.

Current measurements at 21:14 CT: 61% and Charging; battery 33.9–34.0 °C; CPU package 67–76 °C; cycle_count 7; gauge charge_full 8975 mAh versus 8800 mAh design (a non-credible 102% gauge health); one 80→20 discharge took 70.4 minutes at about 17.8 W; 20→62% charging took 65 minutes and projected 20→80% at about 90–95 minutes. The observed band implies roughly 8–9 switch cycles per day.

Decision pending  James still needs to choose whether to keep 20/80, narrow the band to 30/80 or 40/80, use a gentler mid-band such as 45/65, or run mostly on AC with an occasional exercise cycle. Because this is an aftermarket pack with no charge-control-end threshold, replacement or AC-only operation remains the conservative option.

## 4.4 wrk-02 no-sleep

So that wrk-02 keeps serving the cluster with the lid closed or the charger off, logind ignores lid and idle events. /etc/systemd/logind.conf.d/10-no-sleep.conf sets:

HandleLidSwitch=ignore
HandleLidSwitchExternalPower=ignore
HandleLidSwitchDocked=ignore
IdleAction=ignore

logind live properties (busctl get-property org.freedesktop.login1 …) all report ignore, before and after the 20:32 reboot.

sleep/suspend/hibernate targets are static (not masked); there is no sleep.conf.d.

The machine stays awake on battery; the only planned power-off is a clean shutdown at about 3% battery. How the ~3% shutdown is implemented is not recorded in the box sources (Appendix B).

Order of defenses: the watcher turns the charger on at 20%, the critical alert fires at 15%, and the ~3% clean shutdown is the last resort if power never comes back.

## 4.5 cmd show battery (SMS command)

Texting cmd show battery from the allowlisted number returns battery and plug state. It is a local target in /home/box/agent-data/shared/n8n-command-router.json (updated 2026-09-30 18:27 CT): the parser returns kind local and the command is answered on our side, never forwarded to n8n. It is read-only and never changes a plug.

| Item | Value |
| --- | --- |
| Usage | cmd show battery [m5¦wrk-02] (aliases: all, both, macbook, wrk02, skynet-wrk-02; default all) |
| Runs | ~/bin/battery-status.sh <system> on the Mac mini (machine 4de696b8-326c-425c-9771-58fd5d99f7be), timeout 20 s; stdout texted back verbatim |
| Data sources | M5 via a forced-command SSH key that only runs pmset -g batt; wrk-02 via SSH /sys/class/power_supply; plugs .74 and .72 via Tasmota |
| Example replies | M5: 77%, charging (AC), plug SkynetM5Power ON / wrk-02: 95%, on battery, plug Skynet-wrk-02-Power OFF |
| Unreachable host | "…: unreachable (…)" sent as-is; if the mini itself fails: "Battery check failed: couldn't reach the Mac mini. Try again shortly." |
| Bad system | cmd show battery toaster → "Unknown system 'toaster'. Usage: cmd show battery [m5¦wrk-02]" |
| Help text | n8n commands: show … Local: cmd show battery [m5¦wrk-02] (battery + plug status). |
| Verbose | battery-status.sh -v adds time remaining / last plug-log action |

Parser tests (/home/box/agent-data/shared/test_parse_n8n_sms_command.py) cover all, both, m5/M5, wrk-02, skynet-wrk-02, "battery: m5", an unknown system, two systems (invalid), no prefix (normal chat), and the misspelled verb (cmd shoe battery → invalid).

## 4.6 AuxPower plug (.73) setup, 20:35–21:02

Found at 192.168.68.73 on Tasmota 8.5.0 with no web password, PowerOnState already 3, timezone at the +01:00 default, ESP8266EX with 1MB flash and 408KB free. Tasmota's upgrade guide requires stepping 8.5.1 → 9.1 → current, and each full .bin.gz (432–469KB) is larger than the free space, so every hop goes through the minimal build first.

| Time | Step | Result |
| --- | --- | --- |
| 20:35 | Config backup (curl http://192.168.68.73/dl) | ~/plug-backup/auxpower-73-8.5.0.dmp (4096 B) on the mini |
| 20:35:42 | 8.5.1 minimal | Confirmed in the web UI at 20:41 (minimal answers Status with Unknown, so the helper was changed to check the web page) |
| 20:41:48 | 8.5.1 full | 20:43:18 Status 2: 8.5.1(tasmota) |
| 20:43:21 | 9.1.0 minimal | 20:44:26 web: MINIMAL 9.1.0 |
| 20:44:30 | 9.1.0 full | 20:45:52 Status 2: 9.1.0(tasmota) |
| 20:45:58 | 15.6.0 minimal | 20:47:22 web: MINIMAL 15.6.0 |
| 20:47:24 | 15.6.0 full (first try) | Empty replies: 15.6.0's Referer check dropped /cm calls. The mini was briefly unreachable and the step timed out |
| 20:58 | Add Referer header to otastep.sh | Status 2 answers again; the final upgrade had not started |
| 20:58:47 | 15.6.0 full | 21:00:34 Status 2: 15.6.0(release-tasmota), Build 2026-08-25T08:56:44, identical to .72 |
| 21:00 | Apply shared config + WebPassword | All values echoed; restart 21:00:29; unauthenticated Power refused, authenticated Power → ON |
| 21:01:16 | OtaUrl → release-15.6.0/tasmota.bin.gz | Matches .72 (had been left on -minimal) |
| 21:01:17–29 | Toggle test | ON → OFF → ON; final state ON (initial ON) |
| 21:02 | AdGuard rewrite + Deco reservation | See 4.7 and 4.8 |
| 21:02:22 | Status 11 | POWER ON |

The hostname changed from tasmota_0700F9-0249 (8.5.0) to tasmota-0700F9-0249. Helper script: mini ~/plug-backup/otastep.sh (OtaUrl <url>; Upgrade 1; poll Status 2 for full builds and the web main page for minimal builds; Referer header added). AuxPower is not part of the battery automation or cmd show battery.

## 4.7 AdGuard bare-name rewrites

Existing AdGuard rewrites are bare hostnames without a suffix (e.g. skynet-cp-01, m5, mini, pi5), so the plugs follow the same pattern. AdGuard's DHCP is off, so names don't come from DHCP. No PTR records were added (same as m5/mini/miniw). The admin password was moved to the mini as a mode-600 netrc and deleted afterwards on both sides.

| Name | Answer | Added | Check |
| --- | --- | --- | --- |
| skynetm5power | 192.168.68.74 | ~19:20–19:25, HTTP 200 | dig +short @192.168.68.151 skynetm5power → 192.168.68.74 |
| skynet-wrk-02-power | 192.168.68.72 | ~19:20–19:25, HTTP 200 | dig → 192.168.68.72 |
| auxpower | 192.168.68.73 | 21:02, HTTP 200 (enabled:true) | dig → auxpower. 10 IN A 192.168.68.73 |

Rewrite count: 10 → 12 → 13. User filtering rules unchanged (5 PTR $dnsrewrite rules: cp-02, cp-03, wrk-02, wrk-03, pi5).

## 4.8 Deco BE67 DHCP reservations via the hidden local API

CRITICAL — Do not reserve 192.168.68.40 in Deco
192.168.68.40 is the kube-vip API VIP. It must never be placed in the Deco reservation list; reserving it can collide with the cluster control-plane address.

The Deco web UI has no address-reservation page, and the tplinkrouterc6u Deco client has no reservation methods. A first read-only pass got only "no such callback" or 404 on every guessed endpoint, and at that point the conclusion was that reservations could only be made in the Deco app (More > Advanced > Address Reservation). At 19:54 the app-only form admin/client?form=addr_reservation, documented by the open-source tplink-deco-api project, answered a read-only getlist: 21 entries, max 256. The plugs were not reserved and .72/.74 were free.

The adds (approved by James) used mini ~/deco-tmp/addres.py: one session per entry with a getlist baseline, ONE add, getlist again, a diff, and logout in finally. Two request shapes were rejected with error_code 1 "invalid args" and changed nothing: flat params {mac, ip}, and flat params with a base64 name. The working shape wraps the entry in reservation_list:

{"operation":"add","params":{"reservation_list":[{"mac":"AA-BB-CC-DD-EE-FF","ip":"192.168.68.x"}]}}   ->   {"error_code":0}

| Time | Entry | Count | Result |
| --- | --- | --- | --- |
| ~20:00 | 4C-11-AE-00-5E-86 → 192.168.68.74 (SkynetM5Power) | 21 → 22 | MISSING [] EXTRA [(…5E-86, .74)], VERIFIED |
| ~20:00 | 4C-11-AE-00-64-E2 → 192.168.68.72 (Skynet-wrk-02-Power) | 22 → 23 | VERIFIED |
| 21:02 | C8-2B-96-07-00-F9 → 192.168.68.73 (AuxPower) | 23 → 24 | VERIFIED, confirmed by a separate getlist |

Only adds were sent: no modify, remove, reboot, DHCP-pool or Wi-Fi changes. Entries hold only mac and ip; device names in the app come from the client list. 192.168.68.40 is the kube-vip API address and stays unreserved (it must never be a Deco reservation). The router password was moved as a mode-600 file in a mode-700 folder and deleted after every run (verified gone). The procedure is captured in the skill deco-dhcp-address-reservations-local-api.

Final reservation list (24 entries; names from the Deco client list where known):

| MAC | IP | Device |
| --- | --- | --- |
| 18-60-24-27-BC-90 | 192.168.68.101 | skynet-cp-01 |
| 10-E7-C6-15-C6-8E | 192.168.68.102 | skynet-cp-02 |
| F4-39-09-46-8D-96 | 192.168.68.103 | skynet-cp-03 |
| F4-39-09-44-B8-9E | 192.168.68.111 | skynet-wrk-01 |
| C8-2A-14-02-91-23 | 192.168.68.112 | skynet-wrk-02 |
| 38-C9-86-2E-37-A8 | 192.168.68.113 | skynet-wrk-03 (wired enp3s0f0) |
| 88-A2-9E-6B-BC-FF | 192.168.68.151 | AdGuard host (LAN DNS) |
| B8-27-EB-0C-79-92 | 192.168.68.133 | Unknown (Raspberry Pi OUI) |
| 8C-26-AA-E3-DC-DD | 192.168.68.75 | Unknown |
| 20-A5-CB-D2-46-91 | 192.168.68.70 | Unknown |
| C8-FF-77-34-0F-14 | 192.168.68.68 | Unknown |
| C8-FF-77-1B-D0-EF | 192.168.68.67 | Unknown |
| 28-F0-76-2D-B7-06 | 192.168.68.66 | Unnamed in the Deco log; the wrk-03 log shows this is wrk-03's Wi-Fi NIC wlp2s0 (6.3) |
| 44-61-32-E4-85-F3 | 192.168.68.63 | Unknown |
| C0-C7-DB-15-8C-90 | 192.168.68.62 | Unknown |
| DE-F5-EA-C1-AC-CD | 192.168.68.61 | Unknown |
| B4-B6-86-17-6E-14 | 192.168.68.60 | Unknown |
| CC-6A-10-4C-BE-AA | 192.168.68.59 | Unknown |
| D4-8A-FC-5A-CE-34 | 192.168.68.57 | Unknown |
| C8-69-CD-33-89-4C | 192.168.68.54 | Unknown |
| B0-4A-39-CF-36-17 | 192.168.68.53 | Unknown |
| 4C-11-AE-00-64-E2 | 192.168.68.72 | Skynet-wrk-02-Power (new) |
| C8-2B-96-07-00-F9 | 192.168.68.73 | AuxPower (new) |
| 4C-11-AE-00-5E-86 | 192.168.68.74 | SkynetM5Power (new) |

# 5. wrk-02 apt upgrade

20:22 (read-only check) and 20:31:46: sudo apt-get update; sudo DEBIAN_FRONTEND=noninteractive apt -y full-upgrade → exit 0, 0 upgraded.

"Not upgrading yet due to phasing … Not Upgrading: 4": glycin-loaders, glycin-thumbnailers, libglycin-2-0, libglycin-gtk4-2-0 → 2.1.5+ds-0ubuntu0.2, phased at 30% (apt-cache policy confirms). Left to the phased rollout, not overridden.

The pending reboot (reboot-required.pkgs: gnome-shell) was cleared by the rolling reboot in 7.3.

# 6. skynet-wrk-03: 24.04.5 → 26.04.1 Desktop

wrk-03 was the only node still on Ubuntu 24.04.5 (the others were already on 26.04.1, kernel 7.0.0-34). Phase 1 (drain, backups, prep) ran 19:16–19:26. Phase 2 (network fix, release upgrade, desktop, PHP, uncordon) was approved by James at 19:30 and finished at 20:20. Commands ran from the Mac mini.

## 6.1 Pre-check and drain

19:16: wired NIC enp3s0f0 192.168.68.113/22 (default metric 100), Wi-Fi wlp2s0 .66 (metric 600), uptime 3d17h.

Longhorn: 3 replicas on wrk-03 (pvc-482c2abb, pvc-5a721c3c, pvc-644a8ee8 [mealie]). Every volume had 2 other replicas, so node-drain-policy block-if-contains-last-replica would not block. pvc-691cc477 detached/unknown (pre-existing).

Non-DaemonSet pods: Longhorn csi-attacher/provisioner/snapshotter, mealie, prometheus-0, instance-manager.

19:16:27 cordon + drain (--ignore-daemonsets --delete-emptydir-data --timeout=20m). The instance-manager eviction hit its PDB 4× (retried every 5 s) until mealie's volume detached; drained at 19:16:48 (~21 s), no force.

Mealie moved to wrk-01, prometheus-0 to wrk-02; the three volumes went degraded (2/3) as expected.

## 6.2 Backups

/var/backups/pre-2604-20260930 (root, mode 700, files 600), total 4.0M, streamed to the mini ~/Backups/wrk-03-pre-2604-20260930 with sudo tar; SHA-256 checksums match.

| File | Size / content |
| --- | --- |
| etc.tar.gz | 1,415,820 B (3289 entries) |
| etc-php.tar.gz | 58,767 B |
| etc-rancher.tar.gz | 330 B (k3s node-token + node password; secret) |
| k3s-agent.service / .env | 797 B / 82 B (.env is secret) |
| dpkg-selections.txt / apt-manual.txt | 1877 packages / 85 manual |
| mysql-all.sql | 2,603,696 B, "Dump completed on 2026-09-30 19:17:36", 2 CREATE DATABASE |
| Added in Phase 2 | network-pre-phase2.tar.gz (6.1 KB), nginx-pre-php85.tar.gz, k3s-agent.service.pre-nodeip; copied to the mini at 20:21:13, SHA-256 match (mini folder 4.0M, 11 files) |

## 6.3 Prep: apt full-upgrade and the network problem

19:17:56 full-upgrade: 32 upgraded, 1 held (thermald, phased 20%); no kernel image change. Reboot 19:19:36, SSH back 19:21:46.

After the reboot the node registered with InternalIP 192.168.68.66 (Wi-Fi). enp3s0f0 had its address but no routes, so k3s-agent autodetected the only default route (Wi-Fi). NetworkManager also logged "conflict detected for IP address 192.168.68.113 with host 28:F0:76:2D:B7:06". That MAC is wrk-03's own Wi-Fi NIC answering ARP for .113 (ARP flux, arp_ignore=0/arp_announce=0), not a foreign device. The root cause of the missing routes was not identified.

19:23:03 nmcli device reapply enp3s0f0 made it worse (address lost, .113 unreachable; the cluster was unaffected). 19:23:50 nmcli con up "Wired connection 1" restored DHCP and routes.

sysctl.conf: the invalid line sysctl -w net.ipv4.ip_unprivileged_port_start=80 was commented out (backup /etc/sysctl.conf.bak-20260930); the live value stays 1024.

k3s pin: '--node-ip' '192.168.68.113' '--flannel-iface' 'enp3s0f0' added to the k3s-agent ExecStart (backup k3s-agent.service.pre-nodeip). After the restart: InternalIP .113, flannel.1 local .113 dev enp3s0f0, overlay ping from cp-01 to 10.42.3.1 OK.

Third-party repos untouched; no holds. Release check: Prompt=lts, "New release '26.04.1 LTS' available". Only ubuntu-desktop-minimal was installed (not the full ubuntu-desktop).

## 6.4 Network fix (Phase 2)

Persistent profile wired-enp3s0f0 (DHCP, route-metric 100, autoconnect priority 100; UUID c22abf1f-4aec-4249-be51-d90eadf3fc63, stored as /etc/netplan/90-NM-c22abf1f-….yaml).

In-memory "Wired connection 1" deleted; the wired MAC is in no-auto-default.state so it won't be regenerated.

/etc/sysctl.d/90-arp-multihome.conf: arp_ignore=1, arp_announce=2 (all/default); gratuitous ARP sent. .113 now resolves to the wired MAC 38:c9:86:2e:37:a8 on the mini, cp-01 and wrk-02.

Proof reboot 19:30:53 → 19:33:02: wired default route metric 100 (Wi-Fi 600 fallback), no conflict message, node at .113.

## 6.5 Release upgrade and desktop

Pre-checks: / 297G free, apt-get check and dpkg --audit clean, Ubuntu Pro attached (esm-apps, esm-infra, livepatch).

Temporary /etc/apt/apt.conf.d/99zz-release-upgrade-confold (--force-confdef/--force-confold) to keep modified conffiles; removed at 20:18.

19:33:59 do-release-upgrade -f DistUpgradeViewNonInteractive in tmux session relupg (log /var/log/dist-upgrade-wrk03-20260930.log). EXIT=0 at 20:08:43 (~35 min), no dpkg errors or conffile prompts. sudo was briefly missing 19:45–19:57 (sudo → sudo-rs switch; /usr/bin/sudo.ws used for monitoring); SSH refused for ~2 min while openssh-server was reconfigured.

Packages 1817 → 1745 (276 gone, 204 new). Removed: php8.3-*, php-codecoverage, rancher-desktop (+ qemu/ovmf/seabios deps), gnome-bluetooth, libmarisa0, the xorg/xserver-xorg-* set (26.04 GNOME is Wayland-only; xwayland kept), several GNOME extensions and apps, network-manager-gnome, networkd-dispatcher, python3.12. (sort|comm gives wrong results on 26.04's uutils coreutils, so a Python set-diff was used.)

Protected set intact: cryptsetup, mariadb 1:11.8.6, nfs-common, open-iscsi, /usr/local/bin/k3s, network-manager 1.54.3, netplan.io 1.2. /etc/sysctl.conf no longer exists (saved as .dpkg-bak; it had no active lines).

Reboot 20:10:56 → 20:13:14: Ubuntu 26.04.1 LTS, kernel 7.0.0-34-generic, no failed units, network correct.

20:14:31 apt-get install ubuntu-desktop → 139 new (ubuntu-desktop 1.570.4, gnome-shell 50.1, gdm3 50.1). 20:16:41 autoremove of 15 leftover 24.04 libraries.

## 6.6 PHP / nginx / MariaDB

The upgrade removed php8.3 (26.04 ships 8.5). 20:17:15 installed php-fpm php-cli php-curl php-gd php-mbstring php-mysql php-readline php-xml → PHP 8.5.4 with OPcache built in.

Copied the custom 60-custom.ini to /etc/php/8.5/fpm/conf.d (memory_limit 512M, upload/post 20M, opcache.memory_consumption 256, interned_strings_buffer 32). Pool config identical apart from the socket path.

nginx default.conf and texastest.conf now use the version-independent unix:/run/php/php-fpm.sock; nginx -t OK; tests returned "8.5.4 512M opcache mysqli" and HTTP 200 for texastest.

Locally nginx answers only over IPv6, because IPv4 :80 is DNAT'd by k3s svclb-traefik (pre-existing, not a regression).

MariaDB 1:11.8.6 already upgraded; databases information_schema, mysql, performance_schema, sys, wordpress (12 tables).

## 6.7 Uncordon and final state

Longhorn had already rebuilt the three degraded volumes at 19:26:52–58 (after replica-replenishment-wait-interval 600 s).

20:18:50 kubectl uncordon skynet-wrk-03; Longhorn node Ready/Schedulable. wrk-03 holds 0 replicas (Longhorn does not move them back by itself). Mealie stays on wrk-01.

Active: k3s-agent, iscsid, mariadb, php8.5-fpm, nginx, NetworkManager, gdm; no failed units; 0 upgradable; sources resolute + ESM; rancher/wezterm repos .disabled.

# 7. Rolling reboots: cp-02, wrk-02, cp-03

CRITICAL — Never drain with --force; snapshot etcd before control-plane reboots
A blocked drain stops the sequence for review. Always take and verify an etcd snapshot before rebooting a control-plane node, then proceed one node at a time only after Ready, etcd, and storage health are confirmed.

cp-02, cp-03 and wrk-02 had /var/run/reboot-required set (gnome-shell). The first attempt at 20:21 (order wrk-02 → cp-02 → cp-03) stopped before any change: a server-side drain dry run on wrk-02 refused the bare DevPod pod devpod/devpod-default-de-115ea (no controller), and the rule is never to use --force. The CNPG primary was also on cp-03, where the primary PDB allows 0 disruptions. James approved a retry at 20:26 in the order cp-02 → wrk-02 (DevPod stop approved) → cp-03 (CNPG switchover first).

## 7.1 Control-plane checks and etcd snapshot

VIP /readyz ok; readyz?verbose etcd ok; /readyz/etcd and /readyz/etcd-readiness ok on .101/.102/.103.

etcdctl member list was not possible (only the snap etcdctl 3.4.36 is installed, and it cannot read the k3s etcd TLS folder); certs were not copied elsewhere to get round it. Membership and leader were read from etcd's local metrics listener instead (127.0.0.1:2381 /health and /metrics) with a small Python script (/tmp/etcdchk.py on the mini).

20:26:13 sudo k3s etcd-snapshot save --name pre-reboot-20260930 on cp-01 → pre-reboot-20260930-skynet-cp-01-1790817975, 35,745,824 B, kept on cp-01. ("Unknown flag … found in config.yaml" warnings are harmless.)

## 7.2 Timings

| Step | cp-02 (.102) | wrk-02 (.112) | cp-03 (.103) |
| --- | --- | --- | --- |
| apt full-upgrade | 20:26:22, 14 pkgs (code 1.140.0, gstreamer, gvfs, openvpn, libauthen-sasl-perl) | 20:31:46, 0 (4 glycin phased) | 20:40, 13 pkgs |
| Cordon | 20:27:05 | 20:32:03 | 20:40:34 |
| Extra step | None | DevPod pod deleted 20:32:03–20:32:09 | CNPG switchover 20:39:33–20:39:53 (before cordon) |
| Drain done | 20:27:08 (no PDB blocks) | 20:32:26 (one PDB retry) | 20:41:05 (no PDB blocks) |
| Reboot | 20:27:13 | 20:32:30 | 20:41:09 |
| SSH back | 20:29:07 (~1m54s) | 20:35:45 (~3m15s) | 20:43:04 (~1m55s) |
| Verified Ready | 20:29:57 | 20:36:28 | 20:43:53 |
| Uncordon | 20:30:30 | 20:36:34 | 20:44:06 |
| Healthy after | 20:31:15 | Longhorn healthy 20:38:41 | Longhorn healthy 20:46:06 |
| VIP / etcd leader | VIP cp-02 → cp-01 by 20:27:28; leader stayed cp-03 | n/a | Leader cp-03 → cp-01 by 20:41:24; VIP stayed on cp-01 |

Each verify: uptime, kernel 7.0.0-34-generic, no reboot-required, k3s/k3s-agent active, no failed units, kubectl node Ready,SchedulingDisabled; for servers also /readyz/etcd ok ×3 and 3 healthy etcd members with a leader.

## 7.3 wrk-02 and the DevPod workspace devops2

The bare pod is DevPod workspace devops2 (UID default-de-115ea, image ghcr.io/loft-sh/dockerless:0.2.0), created at 18:49 from another machine. The mini's devpod v0.6.15 has no workspaces or providers, so it could not be stopped cleanly and the pod was deleted instead (approved).

PVCs kept: devpod-default-de-115ea = pvc-482c2abb (Longhorn 40Gi, PV reclaim Retain) and devpod-scratch = pvc-b2ba5fe5 (smb-32tb-devstorage 500Gi RWX). Both still Bound; pvc-482c2abb is now detached.

After uncordon, three volumes were degraded (their wrk-02 replicas failed during the drain) and were back to 3/3 by 20:38:41. prometheus-0 moved to cp-02.

battery-plug.timer and 10-no-sleep.conf verified after the reboot (4.3, 4.4).

Restart later with devpod up devops2 from the machine whose devpod CLI has the workspace.

## 7.4 CNPG switchover before cp-03

kubectl-cnpg was not installed. brew install kubectl-cnpg failed with Homebrew permission errors (ownership on the mini was not changed). Installed the official v1.30.0 release (matches the operator) to ~/.local/bin/kubectl-cnpg after shasum -a 256 -c OK; not on the default PATH.

Before: healthy 3/3, primary homelab-postgres-2 on cp-03 since 2026-09-27 02:14 CT, timeline 4; -1 (cp-02) and -3 (cp-01) streaming, quorum, lag 0. No continuous backup configured.

20:39:33 kubectl cnpg promote homelab-postgres homelab-postgres-1 -n database → healthy at 20:39:53 (~20 s). New primary homelab-postgres-1 on skynet-cp-02 (promoted 20:39:36), timeline 5.

After the cp-03 reboot: healthy 3/3, -2 (cp-03) and -3 (cp-01) streaming, lag 0. Primary not switched back (not needed).

# 8. Final cluster state (20:46:12 CT)

| Node | Role | Uptime at check | Notes |
| --- | --- | --- | --- |
| skynet-cp-01 (.101) | server | 3d18h | Holds VIP 192.168.68.40; etcd leader; etcd snapshot pre-reboot-20260930 |
| skynet-cp-02 (.102) | server | 17m | CNPG primary homelab-postgres-1; prometheus-0 |
| skynet-cp-03 (.103) | server | 3m | homelab-postgres-2 (replica) |
| skynet-wrk-01 (.111) | agent | 3d18h | mealie |
| skynet-wrk-02 (.112) | agent | 11m | traefik; battery-plug.timer; no-sleep |
| skynet-wrk-03 (.113) | agent | 33m | Ubuntu 26.04.1 Desktop; node-ip pinned |

6/6 Ready, all Ubuntu 26.04.1 LTS, kernel 7.0.0-34-generic, k3s v1.36.3+k3s1, containerd 2.3.2-k3s2. No reboot-required and no failed units anywhere.

etcd: 3 healthy members, leader cp-01. VIP /readyz ok.

Longhorn: pvc-5a721c3c, pvc-644a8ee8, pvc-7e40275f attached/healthy; pvc-482c2abb detached (devops2 stopped, PVC kept); pvc-691cc477 detached/unknown (pre-existing). Longhorn nodes 6/6 Ready/Schedulable.

No pods outside Running/Completed. cloudflared on cp-01 + wrk-02.

# 9. Security and credential handling

The AdGuard and Deco passwords live only as env vars on the box. They were moved to the mini as mode-600 files (netrc / dp) in a mode-700 folder, used, then deleted on both sides and checked gone. No secrets are left in ~/deco-tmp (grep for netrc content: 0 files).

Every Deco session logged out in finally.

The Tasmota web password is in plaintext inside the plug URLs in the M5 AppleScript and battery-plug.sh (and in earlier log lines); shown here as <plug-password>.

The BlueBubbles password is kept in /etc/battery-plug.env, parsed (not sourced) and passed to curl with --config.

The etc-rancher.tar.gz and k3s-agent.service.env backups contain the k3s node token/password (root-only, mode 600).

# 10. Open follow-ups and leftovers

## 10.1 Power / network

M5 BatteryAlert: the box copy still sends notifications/iMessages at threshold crossings; confirm the live M5 copy is failure-only as intended.

Name the 15 unknown Deco reservations (.53–.75, .133) if wanted; entries carry no name field.

Deco modify/remove on addr_reservation are untested; test carefully, with approval, before relying on them.

Left on the mini (no secrets): ~/deco-tmp (venv, read.py, probe.py, getlist.py, addres.py, downloaded UI JS, dl/, run74.out, run74b.out, run74c.out, run72.out, final.json, gl-before.out) and ~/plug-backup (auxpower-73-8.5.0.dmp, otastep.sh).

No PTR records for the plugs (by design, same as m5/mini).

## 10.2 Cluster

DevPod workspace devops2 is stopped; restart with devpod up devops2 from the machine that owns it.

kubectl-cnpg 1.30.0 is at ~/.local/bin on the mini (export PATH=$HOME/.local/bin:$PATH).

Homebrew on the mini has permission problems (brew install failed; the suggested chown was not done).

wrk-02: 4 glycin updates held by the phased rollout.

etcd snapshot pre-reboot-20260930-skynet-cp-01-1790817975 kept on cp-01.

CNPG has no continuous backup configured (shown in cnpg status).

wrk-03 has 0 Longhorn replicas; prometheus-0 and mealie were not moved back.

Root cause of the missing wired routes on wrk-03 after the first reboot not identified (worked around by the persistent profile).

## 10.3 wrk-03 cleanup

/root/run-dist-upgrade.sh; /var/log/dist-upgrade-wrk03-20260930.log and /var/log/dist-upgrade/; /tmp/ubuntu-desktop-install.log, /tmp/php85-install.log, /tmp/before.txt, /tmp/after.txt, /tmp/i83, /tmp/i85.

/etc/sysctl.conf.bak-20260930 and /etc/sysctl.conf.dpkg-bak (both inactive).

73 rc packages (incl. php8.3-*, /etc/php/8.3 kept) could be purged; 372M of debs in /var/cache/apt/archives (apt clean); the old kernel 7.0.0-31 and the transitional linux-generic-hwe-24.04 meta are still installed.

rancher-desktop (+ qemu deps) removed; rancher and wezterm repos .disabled (wezterm 20240203-110809-5046fc22 still installed).

Backups in /var/backups/pre-2604-20260930/ (all copied to the mini, checksums match).

# Appendix A. Commands

As run on 2026-09-30 unless marked otherwise. LAN commands ran on the Mac mini (user skynet). Secrets are placeholders: <plug-password> (Tasmota web), ~/.adg_netrc or ~/deco-tmp/s/agnetrc (AdGuard netrc, mode 600, deleted after use), s/dp (Deco password file, deleted after use).

## A.1 Tasmota plugs

# Status / MAC
curl -s 'http://192.168.68.72/cm?user=admin&password=<plug-password>&cmnd=Status%205'
curl -s 'http://192.168.68.74/cm?user=admin&password=<plug-password>&cmnd=Status%205'
curl -s 'http://192.168.68.73/cm?cmnd=Status%200'     # AuxPower before a password was set

# AuxPower config backup and OTA step path (each hop: minimal first, then full)
curl -s -o ~/plug-backup/auxpower-73-8.5.0.dmp http://192.168.68.73/dl
curl -sI http://ota.tasmota.com/tasmota/release-{8.5.1,9.1.0,15.6.0}/tasmota{,-minimal}.bin.gz
#   OtaUrl http://ota.tasmota.com/tasmota/release-8.5.1/tasmota-minimal.bin.gz ; Upgrade 1
#   OtaUrl .../release-8.5.1/tasmota.bin.gz ; Upgrade 1
#   OtaUrl .../release-9.1.0/tasmota-minimal.bin.gz ; Upgrade 1
#   OtaUrl .../release-9.1.0/tasmota.bin.gz ; Upgrade 1
#   OtaUrl .../release-15.6.0/tasmota-minimal.bin.gz ; Upgrade 1
#   OtaUrl .../release-15.6.0/tasmota.bin.gz ; Upgrade 1
# (driven by ~/plug-backup/otastep.sh; 15.6.0 needs the Referer header until SetOption128 1)
curl -s -H 'Referer: http://192.168.68.73/' 'http://192.168.68.73/cm?cmnd=Status%202'
sed -i '' "s|curl -s |curl -s -H 'Referer: http://192.168.68.73/' |g" ~/plug-backup/otastep.sh

# Shared config (sent with the Referer header), then the password
#   SetOption128 1 ; DeviceName AuxPower ; FriendlyName1 AuxPower ; Timezone 99
#   TimeDst 0,2,3,1,2,-300 ; TimeStd 0,1,11,1,2,-360 ; Latitude 29.72 ; Longitude -98.07 ; PowerOnState 3
curl 'http://192.168.68.73/cm?cmnd=WebPassword%20<plug-password>'
curl 'http://192.168.68.73/cm?user=admin&password=<plug-password>&cmnd=Power'
#   OtaUrl http://ota.tasmota.com/tasmota/release-15.6.0/tasmota.bin.gz
#   Toggle test: Power ; Power Off ; Power ; Power On ; Power ; Status 11

## A.2 Battery automation

# M5 (BatteryAlert applet): battery read and plug commands
pmset -g batt
curl -s -m 5 'http://192.168.68.74/cm?user=admin&password=<plug-password>&cmnd=Power%20On'
curl -s -m 5 'http://192.168.68.74/cm?user=admin&password=<plug-password>&cmnd=Power%20Off'
tail ~/Scripts/battery_alert.log

# wrk-02 watcher (on wrk-02)
systemctl is-active battery-plug.timer k3s-agent
systemctl list-timers battery-plug.timer
cat /sys/class/power_supply/BAT0/capacity /sys/class/power_supply/ADP1/online
sudo tail /var/log/battery-plug.log
sudo cat /var/lib/battery-plug/state

# wrk-02 no-sleep
cat /etc/systemd/logind.conf.d/10-no-sleep.conf
busctl get-property org.freedesktop.login1 /org/freedesktop/login1 org.freedesktop.login1.Manager HandleLidSwitch
#   (same for HandleLidSwitchExternalPower, HandleLidSwitchDocked, IdleAction)
sudo dmidecode -s system-product-name     # MacBookPro8,3

# cmd show battery (what the SMS command runs on the mini)
~/bin/battery-status.sh all     # or m5 | wrk-02 ; -v adds time remaining / last plug action

# wrk-02 battery cycle watchdog
sudo systemctl enable --now battery-cycle-watchdog.timer
systemctl list-timers battery-cycle-watchdog.timer battery-plug.timer
sudo battery-cycle-watchdog status
sudo battery-cycle-watchdog run --log-only
sudo battery-cycle-watchdog backfill          # use --force only when intentionally reseeding
sudo battery-cycle-watchdog selftest
sudo cat /etc/battery-cycle-watchdog.conf
sudo tail -n 20 /var/log/battery-cycles.csv
sudo journalctl -u battery-cycle-watchdog.service -n 20

The log abbreviates the busctl call; the object path and interface are shown here in standard form.

## A.3 AdGuard rewrites

curl -s -o /dev/null -w '%{http_code}' http://192.168.68.151:{80,3000}/control/status
curl -s --netrc-file ~/.adg_netrc http://192.168.68.151/control/rewrite/list
curl -s --netrc-file ~/.adg_netrc -H 'Content-Type: application/json' -X POST \
  -d '{"domain":"skynetm5power","answer":"192.168.68.74"}' http://192.168.68.151/control/rewrite/add
curl -s --netrc-file ~/.adg_netrc -H 'Content-Type: application/json' -X POST \
  -d '{"domain":"skynet-wrk-02-power","answer":"192.168.68.72"}' http://192.168.68.151/control/rewrite/add
curl --netrc-file ~/deco-tmp/s/agnetrc -H 'Content-Type: application/json' \
  -d '{"domain":"auxpower","answer":"192.168.68.73","enabled":true}' http://192.168.68.151/control/rewrite/add
dig +short @192.168.68.151 skynetm5power
dig +short @192.168.68.151 skynet-wrk-02-power
dig @192.168.68.151 auxpower
rm -f ~/.adg_netrc ~/deco-tmp/s/agnetrc

## A.4 Deco reservations

route -n get default                    # gateway 192.168.68.1 = Deco main unit
python3 -m venv ~/deco-tmp/venv && ~/deco-tmp/venv/bin/pip install tplinkrouterc6u
mkdir -p ~/deco-tmp/s && chmod 700 ~/deco-tmp/s     # password file s/dp, mode 600
cd ~/deco-tmp
venv/bin/python getlist.py              # read-only getlist
venv/bin/python addres.py 4C-11-AE-00-5E-86 192.168.68.74 21 list SkynetM5Power
venv/bin/python addres.py 4C-11-AE-00-64-E2 192.168.68.72 22 list Skynet-wrk-02-Power
venv/bin/python addres.py C8-2B-96-07-00-F9 192.168.68.73 23 list AuxPower
venv/bin/python getlist.py              # expect 24
rm -P ~/deco-tmp/s/dp; rmdir ~/deco-tmp/s

# Endpoint: POST /cgi-bin/luci/;stok=<tok>/admin/client?form=addr_reservation (AES envelope)
#   {"operation":"getlist"}
#   {"operation":"add","params":{"reservation_list":[{"mac":"AA-BB-CC-DD-EE-FF","ip":"192.168.68.x"}]}}

## A.5 wrk-02 apt

ssh -o BatchMode=yes cyberstar@192.168.68.112 \
  'sudo apt-get update; sudo DEBIAN_FRONTEND=noninteractive apt -y full-upgrade'
ssh -o BatchMode=yes cyberstar@192.168.68.112 'apt list --upgradable; apt-cache policy libglycin-2-0'

## A.6 wrk-03 upgrade

# Drain
kubectl cordon skynet-wrk-03
kubectl drain skynet-wrk-03 --ignore-daemonsets --delete-emptydir-data --timeout=20m

# Backups (on wrk-03, B=/var/backups/pre-2604-20260930)
sudo install -d -o root -g root -m 700 $B; cd /; sudo tar -czf $B/etc.tar.gz etc
sudo cp -a /etc/systemd/system/k3s-agent.service{,.env} $B/
sudo tar -czf $B/etc-rancher.tar.gz etc/rancher; sudo tar -czf $B/etc-php.tar.gz etc/php
sudo sh -c "dpkg --get-selections > $B/dpkg-selections.txt; apt-mark showmanual > $B/apt-manual.txt"
sudo sh -c "mysqldump --all-databases --single-transaction --routines --events > $B/mysql-all.sql"
sudo sh -c "chmod 600 $B/*"
# copy to the mini (mkdir -p + chmod 700 first) and compare checksums
ssh -o BatchMode=yes cyberstar@192.168.68.113 'sudo tar -C $B -cf - .' | tar -C ~/Backups/wrk-03-pre-2604-20260930 -xpf -
shasum -a 256 ~/Backups/wrk-03-pre-2604-20260930/*     # vs sudo sha256sum on wrk-03

# Prep
sudo apt update && sudo DEBIAN_FRONTEND=noninteractive apt -y full-upgrade
sudo systemctl reboot
sudo nmcli con up "Wired connection 1"     # restored wired routes (nmcli device reapply made it worse)
# k3s-agent ExecStart now: k3s agent '--node-name' 'skynet-wrk-03' '--node-ip' '192.168.68.113' '--flannel-iface' 'enp3s0f0'
sudo systemctl daemon-reload && sudo systemctl restart k3s-agent
sudo do-release-upgrade -c

# Network fix (via the Wi-Fi address)
ssh -o HostKeyAlias=192.168.68.113 cyberstar@192.168.68.66 'sudo bash -s'
nmcli con add type ethernet con-name wired-enp3s0f0 ifname enp3s0f0 ipv4.method auto ipv4.route-metric 100 \
  ipv6.method auto ipv6.route-metric 100 connection.autoconnect yes connection.autoconnect-priority 100
nmcli con up wired-enp3s0f0
nmcli con delete "Wired connection 1"
# /etc/sysctl.d/90-arp-multihome.conf: net.ipv4.conf.{all,default}.arp_ignore=1 / arp_announce=2
sysctl -p /etc/sysctl.d/90-arp-multihome.conf
arping -U -I enp3s0f0 192.168.68.113

# Release upgrade (root, in tmux session relupg via /root/run-dist-upgrade.sh)
# /etc/apt/apt.conf.d/99zz-release-upgrade-confold: Dpkg::Options { "--force-confdef"; "--force-confold"; }  (temporary)
DEBIAN_FRONTEND=noninteractive APT_LISTCHANGES_FRONTEND=none NEEDRESTART_MODE=a \
  do-release-upgrade -f DistUpgradeViewNonInteractive >> /var/log/dist-upgrade-wrk03-20260930.log 2>&1
sudo systemctl reboot

# Desktop, cleanup, PHP
sudo DEBIAN_FRONTEND=noninteractive apt-get -y install ubuntu-desktop
sudo apt-get -y autoremove
sudo apt-get -y install php-fpm php-cli php-curl php-gd php-mbstring php-mysql php-readline php-xml
sudo tar -czf /var/backups/pre-2604-20260930/nginx-pre-php85.tar.gz /etc/nginx
sudo cp -a /etc/php/8.3/fpm/conf.d/60-custom.ini /etc/php/8.5/fpm/conf.d/
sudo sed -i 's|unix:/run/php/php8.3-fpm.sock|unix:/run/php/php-fpm.sock|' /etc/nginx/conf.d/{default,texastest}.conf
sudo nginx -t && sudo systemctl restart php8.5-fpm && sudo systemctl reload nginx
sudo rm /etc/apt/apt.conf.d/99zz-release-upgrade-confold

# Uncordon
kubectl -n longhorn-system get volumes.longhorn.io
kubectl uncordon skynet-wrk-03

## A.7 Rolling reboot pre-checks and etcd snapshot

kubectl get nodes -o wide
kubectl -n longhorn-system get volumes.longhorn.io
kubectl get pdb -A
kubectl -n database get pods -L cnpg.io/instanceRole
cat /var/run/reboot-required.pkgs           # on each node
ip -4 -br addr | grep 192.168.68.40         # which server holds the VIP
kubectl --server=https://192.168.68.40:6443 get --raw /readyz
kubectl get --raw '/readyz?verbose'
kubectl --server=https://192.168.68.101:6443 get --raw /readyz/etcd   # also .102, .103 and /readyz/etcd-readiness
ssh -o BatchMode=yes cyberstar@192.168.68.102 'python3 -' < /tmp/etcdchk.py     # etcd :2381 /health + /metrics: members, leader
ssh -o BatchMode=yes cyberstar@192.168.68.101 'sudo k3s etcd-snapshot save --name pre-reboot-20260930'
ssh -o BatchMode=yes cyberstar@192.168.68.101 'sudo k3s etcd-snapshot ls'

## A.8 Node reboot: skynet-cp-02

ssh -o BatchMode=yes cyberstar@192.168.68.102 \
  'sudo apt-get update; sudo DEBIAN_FRONTEND=noninteractive apt -y full-upgrade'
kubectl cordon skynet-cp-02 && \
  kubectl drain skynet-cp-02 --ignore-daemonsets --delete-emptydir-data --timeout=20m
ssh -o BatchMode=yes cyberstar@192.168.68.102 'sudo systemctl reboot'
# wait for SSH, then check Ready
until ssh -o ConnectTimeout=5 -o BatchMode=yes cyberstar@192.168.68.102 true
  do sleep 10; done
ssh -o BatchMode=yes cyberstar@192.168.68.102 \
  'uptime; uname -r; ls /var/run/reboot-required; systemctl is-active k3s; systemctl --failed'
kubectl get node skynet-cp-02               # Ready,SchedulingDisabled
kubectl uncordon skynet-cp-02

## A.9 Node reboot: skynet-wrk-02

ssh -o BatchMode=yes cyberstar@192.168.68.112 \
  'sudo apt-get update; sudo DEBIAN_FRONTEND=noninteractive apt -y full-upgrade'
kubectl -n devpod get pvc                   # confirm the workspace PV is Retain
kubectl cordon skynet-wrk-02
kubectl -n devpod delete pod devpod-default-de-115ea --wait=true --timeout=3m
kubectl -n devpod get pod,pvc               # no pods, both PVCs Bound
kubectl drain skynet-wrk-02 --ignore-daemonsets --delete-emptydir-data --timeout=20m
ssh -o BatchMode=yes cyberstar@192.168.68.112 'sudo systemctl reboot'
until ssh -o ConnectTimeout=5 -o BatchMode=yes cyberstar@192.168.68.112 true
  do sleep 10; done
ssh -o BatchMode=yes cyberstar@192.168.68.112 \
  'uptime; uname -r; ls /var/run/reboot-required; systemctl is-active k3s-agent battery-plug.timer; systemctl --failed'
kubectl get node skynet-wrk-02              # Ready,SchedulingDisabled
kubectl uncordon skynet-wrk-02
kubectl -n longhorn-system get volumes.longhorn.io   # polled every 30s until all attached volumes healthy

The first attempt (20:21:49) only ran kubectl drain skynet-wrk-02 --ignore-daemonsets --delete-emptydir-data --dry-run=server, which refused the bare DevPod pod; nothing was cordoned.

## A.10 CNPG switchover

# kubectl-cnpg 1.30.0 from the GitHub release (brew install kubectl-cnpg failed on permissions)
shasum -a 256 -c     # kubectl-cnpg_1.30.0_darwin_arm64.tar.gz against cnpg-1.30.0-checksums.txt
export PATH=$HOME/.local/bin:$PATH
kubectl cnpg version
kubectl cnpg status homelab-postgres -n database
kubectl cnpg promote homelab-postgres homelab-postgres-1 -n database
kubectl -n database get clusters.postgresql.cnpg.io homelab-postgres \
  -o jsonpath='{.status.currentPrimary} {.status.targetPrimary} {.status.phase}'   # polled every 5s

## A.11 Node reboot: skynet-cp-03

ssh -o BatchMode=yes cyberstar@192.168.68.103 \
  'sudo apt-get update; sudo DEBIAN_FRONTEND=noninteractive apt -y full-upgrade'
kubectl cordon skynet-cp-03 && \
  kubectl drain skynet-cp-03 --ignore-daemonsets --delete-emptydir-data --timeout=20m
ssh -o BatchMode=yes cyberstar@192.168.68.103 'sudo systemctl reboot'
until ssh -o ConnectTimeout=5 -o BatchMode=yes cyberstar@192.168.68.103 true
  do sleep 10; done
ssh -o BatchMode=yes cyberstar@192.168.68.103 \
  'uptime; uname -r; ls /var/run/reboot-required; systemctl is-active k3s; systemctl --failed'
kubectl get node skynet-cp-03               # Ready,SchedulingDisabled
kubectl --server=https://192.168.68.103:6443 get --raw /readyz/etcd
kubectl uncordon skynet-cp-03
kubectl cnpg status homelab-postgres -n database

The SSH wait loop matches the logged method (ssh -o ConnectTimeout=5 … true every 10 s); readiness was checked with kubectl get node. An equivalent one-liner for next time (not run on 09-30): kubectl wait --for=condition=Ready node/<node> --timeout=10m.

## A.12 Final checks

kubectl get nodes -o wide
for ip in 101 102 103 111 112 113; do
  ssh -o BatchMode=yes cyberstar@192.168.68.$ip '[ -e /var/run/reboot-required ] && echo rr; systemctl --failed --no-legend; uptime'
done
kubectl -n longhorn-system get volumes.longhorn.io
kubectl get pods -A | grep -Ev 'Running|Completed'

The loop is a condensed form of the per-node checks in the log.

# Appendix B. Gaps in the sources

There are no dedicated build logs on the box for the M5 BatteryAlert, the wrk-02 battery-plug watcher install, the no-sleep fix, or cmd show battery. Those sections come from the script copies on the box, the n8n config, skill and tests, and the checks in the node-reboots log. Install times and steps aren't recorded.

The ~3% clean-shutdown setting on wrk-02 is not in any box source (only 10-no-sleep.conf is logged); how it is implemented is unknown.

The M5 BatteryAlert box copy sends crossing alerts; whether the live copy was changed to failure-only is not recorded. How the applet is launched on the M5 is not recorded.

~/bin/battery-status.sh (on the mini) is not on the box; its behavior is known only from the skill.

SkynetM5Power's firmware version, and when .72/.74 were first configured, aren't logged.

bbtest.sh's result isn't recorded.

There are no log entries after 21:02 CT on 2026-09-30, so nothing from the early hours of 10-01 is recorded (the only 10-01 timestamps are UTC equivalents of evening CT times).

15 of the 24 Deco reservations have no known device name.

# Appendix C. Related documents

Homelab_DevPod_on_Skynet_Runbook.docx: DevPod provider, workspace PVCs and /scratch (devops2 restart).

Homelab_Local_GitHub_Mirror_and_Storage.docx: storage policy (Longhorn vs SMB, 32TB DevStorage).

Homelab_Weekly_OS_Updates_Runbook.docx: routine node maintenance.

Skills: deco-dhcp-address-reservations-local-api, n8n-sms-commands.

Build logs: /workspace/build-log/2026-09-30-*.md.